Acyl-CoA Profile

Service Code: Acyl-CoA

Summary: Profile 5 long-chain acyl-CoA species by mixed organic solvent extraction.  Extracts are dried, re-suspended and measured by ESI+  ionization on a LC-QQQ  mass spectrometer using MRM methods.  Analytes are reported as uM, with CV's generally ~10%.

Container: 1-2 mL polypropylene centrifuge tube

Normal Volume: Tissue (50-100 mgs); Cells (2E7).  NB: Normally only found in tissues

Minimal Volume: Tissue (30 mg); Cells (~5E6)

Special Handling: If human or primate, note any known presence of infectious agents.

Sample Collection: Snap freeze by liquid nitrogen.  For tissues, resect and snap-freeze as soon as practical in tared centrifuge tube. Provide both sample weight and tared vial weight on sample submission

Reference: Haynes, CA et al. (2008) "Quantitation of fatty-acyl-coenzyme As in mammalian cells by liquid chromatography-electrospray ionization tandem mass spectrometry" J Lipid Res. 49(5), 1113-1125. Haynes, CA (2011) "Analysis of mammalian fatty acyl-coenzyme A species by mass spectrometry and tandem mass spectrometry" Biochim et Biophys Acta 1811(11), 663-668.

Table I: Analytes reported.  Others on special request:

| Analyte | Abbr. | PubCHEM | LOQ(uM) |
| --- | --- | --- | --- |
| CoASH | CoA | 87642 |  |
| Acetyl-CoA | (2:0)-CoA | 444493 |  |
| Propionyl-CoA | (3:0)-CoA | 92753 |  |
| Butyryl-/isobutyryl-( combined)-CoA | (4:0)-CoA | 439173/3036931 |  |
| Isovaleryl-CoA | (5:0i)-CoA | 439855 |  |
| Malonyl-CoA | (mal)-CoA | 10663 |  |
| Methylmalonyl-CoA | (Mmal)-CoA | 123909 |  |
| Succinyl-CoA | (suc)-CoA | 439161 |  |
| NB: Red Font: short-chain CoA's done as separate assay |  |  |  |
| Palmitoyl-CoA | (16:0)-CoA | 52922017 |  |
| Palmitoleoyl-CoA | (16:1)-CoA | 25244394 |  |
| Stearoyl-CoA | (18:0)-CoA | 94140 |  |
| Oleoyl-CoA | (18:1)-CoA | 5497111 |  |
| Linoleoyl-CoA | (18:2)-CoA | 6441626 |  |
| Arachidyl-CoA | (20:0)-CoA |  |  |
| Arachidonyl-CoA | (20:4)-CoA |  |  |
| Behenyl-CoA | (22:0)-CoA |  |  |
|  | (22:6)-CoA |  |  |
| Lignoceryl-CoA | (24:0)-CoA |  |  |
|  | (24:1)-CoA |  |  |
| Cerotyl-CoA | (26:0)-CoA |  |  |
|  | (26:1)-CoA |  |  |

Table II: Internal standards and corresponding analytes

| Internal Standards | Source | Cat# | Analytes quantified | mg/ml |
| --- | --- | --- | --- | --- |
| 17:0 Coenzyme A | Avanti | 870717 |  | 2 |
| 25:0 Coenzyme A | Avanti | 870729 |  | 2 |

Materials

- Agilent 6410 QQQ with 1260 LC unit, chilled autosampler, with standard 54-well autosampler plate

- Vortexer

- Refrigerated centrifuge, capable of 13,000g with eppendorf tube compatible rotor

- Eppendorf Vacufuge

- ice bucket, ice

- Prepared stock solutions of acyl CoAs standards and isotope-labelled C17 CoA (internal standard).

- eppendorf tubes (polypropylene)

- LCMS grade water, acetonitrile, methanol

- ACS Reagent grade chloroform, ammonium acetate, acetic acid, ammonium hydroxide

PROCEDURES:

Preparation of Extraction Solvent

- Solvent 1: Prepare 20 mL of 8:1:1 methanol:water:chloroform (16 mL methanol, 2 mL water, 2 mL chloroform).

- Solvent 2: 2987 uL of 8:1:1, + 3 uL of C17 AcylCoA

Preparation of Standards

Acyl CoA standard (100uM STD mix): (C2:0, C16:0, C16:1, C18:0, C18:1, C18:2, C20:1, C20:2, C20:3, C20:4, [C17:0, C25:0 (IS, 1.96mg/ml)] :

| Standard name/ Concentration | Volume of methanol | Volume of chloroform | Volume of water | Volume of 100 uM STD mix | Volume of extraction solvent 2 |
| --- | --- | --- | --- | --- | --- |
| STD 0 / 0 uM | 200 | 25 | 25 | 0 | 250 |
| STD 1 / 0.1 uM | 200 | 25 | 24.5 | 0.5 | 250 |
| STD 2 / 0.5 uM | 200 | 25 | 22.5 | 2.5 | 250 |
| STD 3 / 2.5 uM | 200 | 25 | 12.5 | 12.5 | 250 |

Plasma/Serum Sample Preparation

- Weigh frozen tissue samples and transfer to labeled eppendorf tube and record weight. Homogenize tissue.

- Add appropriate amount of extraction solvent to the sample in eppendorf tube, vortex to mix.

- Keep all samples on ice for 10 min, vertex again.

- Centrifuge all vials at 4°C and 16,000 xg for 10 minutes. Transfer supernatant to autosampler vial and Analyze by LC-MS

Tissue Sample Preparation

??

Cell Sample Preparation

- Put samples in a box with dry ice.  Put extraction solvent on dry ice.

- Working one plate at a time, remove plate from the cooler and place on a surface of regular ice.

- Clean cell scraper with MeOH and kimwipe.

- Add 540ul of extraction solvent to the plate.

- Scrape cells with cell scraper, then scrape solvent to one corner of the plate.

- Transfer debris to a labeled 2mL eppendorf vial.  Put vial on dry ice, incubate 5 minutes.

- Repeat procedure with all additional eppendorf vials.

- Centrifuge all vials at 15,000g for 10 minutes

- Transfer 100ul of supernatant to an autosampler vial (with insert)

- LCMS Calibration

??

LC-MS procedure

- LC column: Waters C18 column, 2.1mm x 50mm

- Mobile phase A: 5mM ammonium acetate in water, pH 9.9 using NH4OH

- Mobile phase B: 100% acetonitrile

- Gradient: 0min, 10%B, 7min, 100%B, 11min, 100%B, 16min 10%B, flow rate: 0.23/min

- Autosampler: 4°C, 5 uL injection

- Agilent 6410 QQQ: ESI+, Method: QM000 or equivalent

Table III: MRM transitions monitored:

| Analyte | MRM 1 | MRM 2 | Polarity | Quant or qual? |
| --- | --- | --- | --- | --- |
| C16 |  |  | + |  |
| C16:1 |  |  | + |  |
| C17 |  |  | + |  |
| C18 |  |  | + |  |
| C18:1 |  |  | + |  |
| C18:2 |  |  | + |  |
| C20:1 |  |  | + |  |
| C20:2 |  |  | + |  |
| C20:3 |  |  | + |  |
| C20:4 |  |  | + |  |
| C25 |  |  | + |  |
| C2 |  |  | + |  |
| C4 |  |  | + |  |
| C5i |  |  | + |  |
|  |  |  | + |  |