5 min negative:

After sample randomization, 10 μL of extracts were injected into a Thermo Vanquish UHPLC system (San Jose, CA, USA) and resolved on a Kinetex C18 column (150 × 2.1 mm, 1.7 μm, Phenomenex, Torrance, CA, USA) at 450 μL/min through a 5 min gradient from 0 to 100% organic solvent B (mobile phases: A = 95% water, 5% acetonitrile, 1 mM ammonium acetate; B = 95% acetonitrile, 5% water, 1 mM ammonium acetate) in negative ion mode. Solvent gradient: 0-0.5 min 0% B, 0.5-1.1 min 0-100% B, 1.1-2.75 min hold at 100% B, 2.75-3 min 100-0% B, 3-5 min hold at 0% B.

5 minute positive:

After sample randomization, 10 μL of extracts were injected into a Thermo Vanquish UHPLC system (San Jose, CA, USA) and resolved on a Kinetex C18 column (150 × 2.1 mm, 1.7 μm, Phenomenex, Torrance, CA, USA) at 450 μL/min through a 5 min gradient from 5 to 95% organic solvent B (mobile phases: A = water, 0.1% formic acid; B = acetonitrile, 0.1% formic acid) in positive ion mode. Solvent gradient: 0-0.5 min 5% B, 0.5-1.1 min 5-95% B, 1.1-2.75 min hold at 95% B, 2.75-3 min 95-5% B, 3-5 min hold at 5% B.

17 min negative:

After sample randomization, 10 μL of extracts were injected into a Thermo Vanquish UHPLC system (San Jose, CA, USA) and resolved on a Waters ACQUITY HSS T3 2.1 × 150 mm, 1.8 μm at 45 degrees C and sample compartment temperature 7 degrees C using an aqueous phase (A) of 25% acetonitrile and 5 mM ammonium acetate and a mobile phase (B) of 50% isopropanol, 45% acetonitrile and 5 mM ammonium acetate.  The solvent gradient was as follows: 0-1 min 25% B and 0.3 ml/min; 1-2 min 25-50% B and 0.3 mL/min; 2–8 min 50–90% B and 0.3 mL/min; 8–10 min 90–99% B and 0.3 mL/min; 10–14 min hold at 99% B and 0.3 mL/min; 14–14.1 min 99–25% B and 0.3 mL/min; 14.1–16.9 min hold at 25% B and 0.4 mL/min; 16.9–17 min hold at 25% B and resume flow of 0.3 mL/min.

15 min positive:

After sample randomization, 10 μL of extracts were injected into a Thermo Vanquish UHPLC system (San Jose, CA, USA) and resolved on a Waters ACQUITY HSS T3 2.1 × 150 mm, 1.8 μm at 45 degrees C and sample compartment temperature 7 degrees C and using an aqueous phase (A) of 25% acetonitrile and 5 mM ammonium acetate and a mobile phase (B) of 90% isopropanol, 10% acetonitrile and 5 mM ammonium acetate. The solvent gradient was as follows: 0–10 min 30–100% B and 0.325 mL/min; 10–12 min 100% B and 0.325 mL/min; 12–12.5 min 100–30% B and 0.4 mL/min; 12.5–14.9 min 30% B and 0.4 mL/min; 14.9–15 min 30% B and 0.325 mL/min.