Dr. Lampe’s Adipose Tissue

Platform: GCTOF

Adipose Tissue: ~15mg

Tissue was cut on Dry Ice to approximately 15mg (15.35mg). Remaining Tissue was placed back into the dummy tube and stored at -80C in the original box

Extraction Protocol:

- Degased Pre-Chilled Extraction Solvent (3:3:2 Acetonitrile/Isopropanol/ddH2O)

- Kept Extraction Solvent on Ice

- Homogenized Cell Pellets using Cryo GenoGrinder to a fine dust

  - 3-4 balls used

  - It should be noted that the tissue sample was not entirely homogenized. This is likely a consequence of its small size. Alternative extraction methods should be conceived if initial trial shows low abundance of metabolites

- Added 1mL of ice-cold extraction solvent to homogenized cell pellet

- Vortexed briefly for 5 seconds

- Shook for 5 minutes at 4C using the Orbital Mixing Chilling/Heating Plate

- Transfered Supernatant (all) to a new 1.5mL eppendorf tube and placed on ice

- Vortexed for 10seconds

- Shook for 5 minutes at 4C using the Orbital Mixing Chilling/Heating Plate

- Centrifuge samples for 2 minutes at 14,000 rcf

- Aliquoted two 450µL portions of the supernatant into new 1.5mL eppendorf tubes. One for analysis one for a backup sample. Store Backup in -20C.

- Evaporate one 450µL aliquot of the sample in the Labconco Centrivap cold trap concentrator to complete dryness

- Resuspend the dried aliquot with 450µl of 50/50 Acetonitrile/ddH2O (degassed) as a clean-up step to remove lipids

- Centrifuge for 2 minutes at 14,000 rcf

- Remove supernatant to a new 1.5mL eppendorf tube

- Evaporate one 450µL aliquot of the sample in the Labconco Centrivap cold trap concentrator to complete dryness

- Submit for derivatization according to SOP (see attached)

- GCTOF parameter alterations:

  - ***injection volume increased to 1µL rather than 0.5µL. This is due to low abundance of compounds. See Figure Below