NMR Metabolomics Methods

Frozen fecal samples were transferred into numbered homogenizer bead tubes (ceramic) and diluted with D2O (Sigma-Aldrich, St. Louis, MO). Specimens with a mass of less than 100 mg were diluted with 0.5 mL; specimens with a mass of greater than 100 mg were diluted with 1.0 mL. All masses were provided with the shipping manifest. The samples were then homogenized using the MagNA Lyser bead homogenizer for two 30 second pulses at 2000 rpm and then vortexed for an additional 30 seconds. The samples were pelleted by centrifugation at 12000 rcf for 5 minutes. 50 mg of extract were utilized for each specimen, and combined the fecal extract and D2O were a total volume of 630 µL. 70 µL of Chenomx ISTD solution into all tubes. Chenomx ISTD (Chenomx, Edmonton, Alberta, Canada) contains 5mM 4,4-dimethyl-4-silapentane-1-sulfonic acid (DSS, chemical shift indicator), 100 mM imidazole (pH indicator), and 0.2% NaN3 (to inhibit bacterial growth) in D2O. 600 µL was transferred into each NMR tube for analysis.

Phenotypic pooled fecal extract samples were made by combining 50 µl aliquots from each of the study samples belonging to the same phenotype (Dirty and Clean). In addition, a combined phenotypic pooled sample was also prepared by using 630 µl aliquot from each of the phenotypic pooled sample. Pooled NMR samples were prepared as described above and used as quality check (QC) samples.

1H NMR spectra of fecal samples were acquired on a Bruker Avance 700 MHz NMR spectrometer  (located at the David H. Murdock Research Institute at Kannapolis, NC, USA) using a 5 mm cryogenically cooled ATMA inverse probe and ambient temperature of 25℃. A 1D NOESY presaturation pulse sequence (noesypr1d, [recycle delay (RD)-90°-t1-90°-tm-90°-acquire free induction decay (FID)]) was used for data acquisition. For each sample 128 transients were collected into 64k data points using a spectral width of 14.01 kHz (20.14 ppm), 2 s relaxation delay, 100 ms mixing time, and an acquisition time of 2.324 s. The water resonance was suppressed using resonance irradiation during the relaxation delay and mixing time. NMR spectra were processed using ACD NMR Processor 12.0 (ACD Labs, Toronto, CA).  Spectra were zero filled, and Fourier transformed after exponential multiplication with line broadening factor of 0.5 Hz. Phase and baseline of the spectra were manually corrected for each spectrum. Spectra were referenced internally to the DSS signal. The quality of each NMR spectrum was assessed for the level of noise and alignment of identified markers. Spectra were assessed for missing data and underwent quality checks.  NMR bins (0.10-9.0 ppm) were made after excluding DSS, water (4.68-4.88 ppm), and imidazole (7.20-7.28 ppm) using intelligent bucket integration with a 0.04 ppm bucket width and 50% looseness. Integrals of each of the bins were normalized to total integral of each of the spectrum.