LC-MS Metabolomics Methods

Sample Preparation:

50uL of L-tryptophan-d5 (used as an internal standard) and 1500uL of 90:10 methanol:chloroform  were added to each cell pellet of 2x107 cells. For cell pellets that were greater or less than 2x107, L-tryptophan-d5 and 90:10 methanol:chloroform were added in proportional amounts. Samples were vortexed on a multi-tube vortexer for 4 min. at 5000rpm and centrifuged for 4 min. at 16000rcf. Pooled samples were created by combining aliquots from the appropriate study samples to form all-pooled, control-pooled, AZD-pooled, and GSK-pooled samples. For reversed phase chromatography, 600uL of the supernatant was then transferred to new tubes and dried on a SpeedVac for 3 hours at 30C. Samples were reconstituted in 100uL of 95:5 water: methanol, vortexed on a multi-tube vortexer for 2 min. at 5000rpm, and centrifuged for 4 min. at 16000rcf. The supernatant was transferred to autosampler vials. For HILIC chromatography, samples were centrifuged for 4 min. at 16000rcf, and 100uL of supernatant transferred to autosampler vials.

UPLC-MS Methods:

UPLC-MS spectra were collected for all samples. All liquid chromatography was performed on a Waters Acquity UPLC. For reversed phase analysis, 10uL of each sample was injected into an Acquity BEH HSS T3 column (2.1x 100mm x 1.8 um) at 50C. Water with 0.1% formic acid (mobile phase A) and methanol with 0.1% formic acid (mobile phase B) were injected following the RCMRC-RF methods (see the 3. GRAVES MetaData and Analytical Metadata.xlsx file for the flow gradient).  For HILIC analysis, 5uL of each sample was injected into an Acquity BEH Amide column (2.1mm x 150mm x 1.7um) at 40C. 10mM ammonium acetate in 95:5 acetonitrile:water with 0.1% formic acid (mobile phase A) and 10mM ammonium acetate in 50:50 acetonitrile:water (mobile phase B) were injected following the RCMRC-HILIC methods (see the 3. GRAVES MetaData and Analytical Metadat.xlsx file for the flow gradient). Mass spectroscopy analysis was performed using a Synapt G2 Q-TOF. 10uL (reversed phase) or 5uL (HILIC) of each sample was injected into the instrument, and MS data was collected between 70-1000m/z in both positive and negative modes.