Protocol for Liquid NMR Studies of Tissues/Cells Extractions

This Protocol is based on metabolomics studies from Jeremy K Nicholson, et al. Nature Protocols, 2007, 2(11):2692-2703 and Mark R. Viant, et al. Metabolomics, 2007,3(1):55-67.

1. Collection and Storage.

All samples were stored at -80 C until used randomly.

2. Ground/Homogenized (20-100mg tissues)

Tissues could be ground by mortar/pestle manually or disrupted by homogenizer in solvents. It was found that the homogenization method (Sonicator or mechanical homogenizer) gave less inter-sample variability than mortar/pestle ground method. So the homogenization method was adopted.

3. Extraction Procedures (MeOH-CHCl3-H2O)

The extraction procedure with ice-cold MeOH-CHCl3-H2O had the best overall results than other solvents.

3.1 Prepare ice-cold solvents (MeOH, CHCl3 and H2O).

3.2 Weigh the frozen tissue and transfer into a glass vial.

3.3 Homogenize after adding 4ml MeOH and 0.85ml H2O per gram of tissue in ice bath. Vortex the mixture and then add 2ml chloroform per gram of tissue. Vortex again. (Appendix for solvent to tissue ratio is attached)

3.4 Add 2ml chloroform and 2ml H2O per gram of tissue in the mixture and vortex again. At this stage, the mixture will become white cloudy suspension.

3.5 Let the mixture sit in ice or refrigerator for 15min, centrifuge at 8000rpm for 2 min at 4 C. The upper layer is MeOH/H2O phase with polar metabolites; the lower layer is CHCl3 phase with the non-polar metabolites. Protein and cellular debris are between those two layers. If the separation is not good, centrifuge the mixture again.

3.6 Transfer the different layers into different glass vials separately with syringe (1000 µl, hamilton) and label them clearly. Remove the solvents with speedvac or lyophilizer (MeOH/H2O layer) or under nitrogen gas flow (CHCl3 layer).

3.7 The metabolites will be frozen at -80 C after solvents removal.

4. NMR Spectra

4.1 Polar Metabolites

Suspend the tissue extracts with 600 l D2O (Containing 0.5mM TSP or DSS, internal reference), vortex samples and centrifuge at 5000 rpm for 2 min, transfer 550 l D2O solution into NMR tube for measurement (4 C).

0.5 mM TSP D2O solution was prepared as following: TSP (86.4mg, Na+, Deuterated) was dissolved into 10ml D2O, and 1ml of this solution was diluted to 100 ml with D2O in 100 ml volumetric flask. Shake well and seal with parafilm.

4.2 Non-polar Metabolites

Suspend the tissue extract with 600 l CDCl3 (0.03 v/v TMS),vortex samples and centrifuge 5000rpm for 1 min, transfer 550 l CDCl3 solution into NMR tube for measurement (4 C).

5. Data Analysis from NMR Spectra

The intergral of peaks was normalized by internal reference and mass and compared between the control and the diseased groups. PCA was used to explore the metabolomics.

*Glass vials are needed for all steps when using chloroform. Tests have showed polypropylene tube is not good for this protocol.

|  |  |  | Table 1. Solvents-Material ratio for Extraction (0 C) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Material (g) |  | MeOH (ml) |  | H2O (l) | CHCl3 (ml) |  | CHCl3 (ml) | H2O (ml) | CHCl3/MeOH-H2O from Syringe (µl) |
| 1 g |  | 4 |  | 850 | 2 |  | 2 | 2 |  |
| 0.5 g |  | 2 |  | 425 | 1 |  | 1 | 1 |  |
| 0.25 g |  | 1 |  | 213 | 0.5 |  | 0.5 | 0.5 |  |
| 125 mg |  | 0.5 |  | 106 | 0.25 |  | 0.25 | 0.25 |  |
| 62.5 mg |  | 0.25 |  | 53 | 0.125 |  | 0.125 | 0.125 | 220/360 |
| 31.3 mg |  | 0.25 |  | 53 | 0.125 |  | 0.125 | 0.125 | 220/360 |
| 10.0 mg |  | 0.25 |  | 53 | 0.125 |  | 0.125 | 0.125 | 220/360 |