Newman Lab Free & Total Oxylipin Analysis

Materials & Methods

For plasma analysis, free (non-esterified) oxylipins were isolated using a Waters Ostro Sample Preparation Plate (Milford, MA). Sample aliquots (50 μL plasma) were introduced into the plate wells and spiked with a 5 µL anti-oxidant solution (0.2 mg/ml solution BHT/EDTA in 1:1 MeOH:water) and 10 μL 1000nM analytical deuterated surrogates. Acetonitrile (150 μL) with 1% formic acid was forcefully added to the sample and aspirated three times to mix. Samples were eluted into glass inserts containing 10 μL 20% glycerol by applying a vacuum at 15 Hg for 10 min. Eluent was dried by speed vacuum for 35 min at the medium BP setting, before switching to an aqueous setting for an additional 35 min. Once dry, samples were re-constituted with the internal standard 1-cyclohexyl ureido, 3-dodecanoic acid (CUDA) and 1-Phenyl 3-Hexadecanoic Acid Urea (PHAU) at 100 nM (50:50 MeOH:CAN), vortexed 1 min, transferred to a spin filter (0.1 µm, Millipore, Billerica, MA), centrifuged for 3 min at 6ºC at <4500g (rcf), before being transferred to 2 mL LC-MS amber vials. Extracts were stored at -20ºC until analysis by UPLC-MS/MS. The internal standard was used to quantify the recovery of surrogate standards.

The total rat lung lipid extract was isolated as previously reported (1,2). Specifically, rat lung samples (~25 mg) were enriched with 5 mL 0.2 mg/ml butylated hydroxytoluene/EDTA in 1:1 methanol/water (v/v), and enriched with the extraction surrogate dodecatrienoic acid (22:3n3; NuChek Prep). Lipids were then extracted with 10:8:11 cylcohexane/2-propanol/ammonium acetate (v/v/v). Briefly, enriched samples were mixed with cyclopropane/2-propanol, phases were split with ammonium acetate, the organic phase was isolated and the aqueous phase was re-extracted with cyclohexane. The combined organic total lipid extract was reduced to dryness and reconstituted in 100 µL of 1:1 methanol/toluene (v/v). To liberate esterified oxylipins, aliquots of total lipid extract (40 µL) spiked with deuterated oxylipin surrogates were incubated with 100 µL 0.5M sodium methoxide for 1hr at 60 ºC, mixed with 100 µL water, and returned to 60 ºC for 1hr.  Samples were then diluted with 0.5 mL 0.1%AA/5%MeOH, neutralized with 10 µL 20% glacial acetic acid.

Total (free and esterified) oxylipins were isolated from the total lipid extract by solid phase extraction on 10 mg Waters Oasis-HLB cartridges (Milford, MA), as previously reported (3). Prior to extraction, cartridges were washed with 1 column volume ethyl acetate followed by 2 column volumes methanol and conditioned with 2 mL of 95:5 (v/v) water/methanol (MeOH) with 0.1% acetic acid. The column reservoir was spiked with 5 µL anti-oxidant solution, (0.2 mg/ml solution BHT/EDTA in 1:1 MeOH:water), and 10 μL 1000nM analytical surrogates. Sample aliquots (40 μL total lipid extract) were then introduced to the column reservoir and diluted with 1 column volume wash solution (5% MeOH, 0.1% acetic acid), followed by 1 column volumes wash solution (20% methanol, 0.1% acetic acid). SPE cartridges were dried with vacuum @ -25psi for 20 min. Analytes were then extracted into 2 mL autosampler vials (pre-prepared with 10 µL 20% glycerol solution in MeOH) with 0.2 mL MeOH, followed by 0.5 mL Ethyl Acetate, by gravity. Eluent was dried by speed vacuum for 35 min, before they were re-constituted with the internal standard 1-cyclohexyl ureido,3-dodecanoic acid (CUDA) and 1-Phenyl 3-Hexadecanoic Acid Urea (PHAU) at 100 nM (50:50 MeOH:ACN), vortexed 1 min, transferred to a spin filter (0.1 µm, Millipore, Billerica, MA), centrifuged for 3 min at 6ºC at <4500g (rcf), before being transferred to 2 mL LC-MS amber vials. Extracts were stored at -20ºC until analysis by UPLC-MS/MS. The internal standard was used to quantify the recovery of surrogate standards.

Analytes in a 100 μL extract aliquots were separated utilizing a Waters Acquity UPLC (Waters, Milford, MA) with a solvent gradient using modifications of a previously published protocols for oxylipins (4,5) (See Table 1 & 2 for detailed instrument parameters). Samples were held at 10ºC. Separated residues were detected by negative mode electrospray ionization using multiple reaction monitoring on an API 4000 QTrap (AB Sciex, Framingham, MA, USA). Analytes were quantified using internal standard methods and 5 to 10 point calibration curves (r2 ≥ 0.997). Calibrants and internal standards were either synthesized [10,11-DHN, 10,11-DHHep, 10(11)-EpHep and CUDA] or purchased from Cayman Chemical (Ann Arbor, MI), unless otherwise indicated. Loradan Fine Lipids (Malmo, Sweden) provided the linoleate derived triols 9,12,13-TriHOME and 9,10,13- TriHOME. Data was processed utilizing AB Sciex MultiQuant version 3.0.1.

(1) Smedes F (1999). Determination of total lipid using non-chlorinated solvents. Analyst. 124:1711-1718.

(2) Gladine et al (2014). Lipid profiling following intake of the omega 3 fatty acid DHA identifies peroxidized metabolites F4-neuroprostanes as the best predictors of atherosclerosis prevention. PLOS ONE.

(3) Luria A et al (2007). Compensatory mechanism for homeostatic blood pressure regulation in Ephx2 gene-disrupted mice. J Biol Chem. 282:2891-8

(4) Strassburg K et al (2012). Quantitative profiling of oxylipins through comprehensive LC-MS/MS analysis: application in cardiac surgery. Anal Bioanal Chem. 404:1413-26.

(5) Grapov D et al (2012). Type 2 Diabetes Associated Changes in the Plasma Non-Esterified Fatty Acids, Oxylipins and Endocannabinoids. PLoS ONE. 7(11):e48852.

| Table 1. Oxylipin Assay UPLC parameters |  |  |
| --- | --- | --- |
| Time (min) | A% | B% |
| 0 | 90 | 70 |
| 2 | 90 | 80 |
| 5 | 60 | 85 |
| 5.5 | 60 | 90 |
| 13.5 | 20 | 95 |
| 13.75 | 20 | 99 |
| 14.5 | 5 | 99 |
| 14.7 | 5 | 70 |
| 15.2 | 90 | 70 |
| Solvent A = 5 mm NH4COO 0.2% |  |  |
| formic acid; Solvent B = 5 mm |  |  |
| NH4COO 0.2% formic acid in MeOH, |  |  |
| flow rate = 0.25 mL/min, column |  |  |
| 2.1 X 100mm, 1.7 µm BEH C8 (Waters, |  |  |
| Milford, MA), column temp = 60 °C |  |  |

|  | Table 2a. Oxylipin UPLC/ESI QTRAP Analyte and Instrument-specific Parameters * † †† |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Name | Expected RT | Internal Standard (ISTD) Name | Parent ion | Daughter ion |  |
| 1 | PHAU | 2.67 |  | 249.2 | 130.1 |  |
| 2 | 6-keto PGF1a | 3.04 | IS PGD2 | 369.3 | 163.1 |  |
| 3 | d4-6-Keto PGF1a | 3.04 | IS PHAU | 373.3 | 167.1 |  |
| 4 | PGF3a | 3.56 | IS d4-PGF2a | 351.3 | 307.4 |  |
| 5 | PGE3 | 3.69 | IS d4-PGF2a | 349.3 | 269.2 |  |
| 6 | d4-TXB2 | 3.74 | IS PHAU | 373.3 | 173.1 |  |
| 7 | TXB2 | 3.75 | IS d4-TXB2 | 369.3 | 169.1 |  |
| 8 | 9_12_13-TriHOME | 4.22 | IS d4-PGF2a | 329.2 | 211.2 |  |
| 9 | d4-PGF2a | 4.27 | IS PHAU | 357.3 | 197.2 |  |
| 10 | PGF2a - Select | 4.28 | IS d4-PGF2a | 353.3 | 193.2 |  |
| 11 | PGE2 | 4.40 | IS d4-PGD2 | 351.3 | 271.2 |  |
| 12 | PGE1 | 4.58 | IS d4-PGD2 | 353.3 | 317.2 |  |
| 13 | d4-PGD2 | 4.69 | IS PHAU | 355.3 | 275.2 |  |
| 14 | PGD2 | 4.71 | IS d4-PGD2 | 351.3 | 271.2 |  |
| 15 | 15-Keto PGE2 | 4.75 | IS d4-PGD2 | 349.2 | 331.3 |  |
| 16 | Resolvin D1 | 5.23 | IS d4-PGF2a | 375.3 | 121.1 |  |
| 17 | Lipoxin A4 | 5.35 | IS d4-PGF2a | 351.3 | 217.2 |  |
| 18 | LTB5 | 6.40 | IS d4-LTB4 | 333.3 | 195.2 |  |
| 19 | d3-LTE4 | 6.51 | IS CUDA | 441.4 | 336.3 |  |
| 20 | LTE4 | 6.54 | IS d3-LTE4 | 438.4 | 333.3 |  |
| 21 | 15_16-DiHODE | 6.85 | IS d11-14_15-DiHETrE | 311.2 | 235.2 |  |
| 22 | 12_13-DiHODE | 6.93 | IS d11-14_15-DiHETrE | 311.2 | 183.2 |  |
| 23 | 8_15-DiHETE | 6.95 | IS d11-14_15-DiHETrE | 335.3 | 235.2 |  |
| 24 | 9_10-DiHODE | 6.98 | IS d11-14_15-DiHETrE | 311.2 | 201.2 |  |
| 25 | 17_18-DiHETE | 7.23 | IS d11-14_15-DiHETrE | 335.3 | 247.2 |  |
| 26 | 5_15-DiHETE | 7.28 | IS d11-14_15-DiHETrE | 335.3 | 173.1 |  |
| 27 | 6-trans-LTB4 | 7.40 | IS d4-LTB4 | 335.3 | 195.2 |  |
| 28 | 14_15-DiHETE | 7.57 | IS d11-14_15-DiHETrE | 335.3 | 207.2 |  |
| 29 | CUDA | 7.62 |  | 339.4 | 214.2 |  |
| 30 | d4-LTB4 | 7.63 | IS CUDA | 339.3 | 163.1 |  |
| 31 | LTB4 | 7.67 | IS d4-LTB4 | 335.3 | 195.2 |  |
| 32 | 12_13-DiHOME | 7.94 | IS d11-14_15-DiHETrE | 313.3 | 183.2 |  |
| 33 | 10_11-DHHep | 8.02 | IS CUDA | 301.2 | 283.2 |  |
| 34 | 9_10-DiHOME | 8.35 | IS d11-14_15-DiHETrE | 313.3 | 201.2 |  |
| 35 | d11-14_15-DiHETrE | 8.51 | IS CUDA | 348.4 | 207.2 |  |
| 36 | 19_20-DiHDoPA | 8.57 | IS d11-14_15-DiHETrE | 361.3 | 273.20 |  |
| 37 | 14_15-DiHETrE | 8.60 | IS d11-14_15-DiHETrE | 337.3 | 207.2 |  |
| 38 | 11_12-DiHETrE | 9.16 | IS d11-14_15-DiHETrE | 337.3 | 167.1 |  |
| 39 | 9_10-e-DiHO | 9.31 | IS 10_11-DHHep | 315.2 | 297.2 |  |
| 40 | 9-HOTE | 9.42 | IS d4-9(S)-HODE | 293.2 | 171.1 |  |
| 41 | 12(13)-Ep-9-KODE | 9.43 | IS d4-9(S)-HODE | 309.2 | 291.2 |  |
| 42 | 13-HOTE | 9.55 | IS d4-9(S)-HODE | 293.2 | 195.2 |  |
| 43 | 8_9-DiHETrE | 9.70 | IS d11-14_15-DiHETrE | 337.3 | 127.1 |  |
| 44 | 15-deoxy PGJ2 | 9.77 | IS d11-14_15-DiHETrE | 315.2 | 271.2 |  |
| 45 | d6-20-HETE | 9.93 | IS CUDA | 325.3 | 281.2 |  |
| 46 | 15-HEPE | 9.97 | IS d8-12(S)-HETE | 317.2 | 219.2 |  |
| 47 | 20-HETE | 9.97 | IS d6-20-HETE | 319.2 | 275.2 |  |
| 48 | 12-HEPE | 10.33 | IS d8-12(S)-HETE | 317.2 | 179.1 |  |
| 49 | 5_6-DiHETrE | 10.46 | IS d11-14_15-DiHETrE | 337.3 | 145.1 |  |
| 50 | 9-HEPE | 10.53 | IS d4-9(S)-HODE | 317.2 | 167.2 |  |
| 51 | 13-HODE | 10.83 | IS d4-9(S)-HODE | 295.2 | 195.2 |  |
| 52 | 5-HEPE | 10.91 | IS d4-9(S)-HODE | 317.2 | 115.1 |  |
| 53 | d4-9(S)-HODE | 10.95 | IS CUDA | 299.2 | 172.1 |  |
| 54 | 9-HODE | 11.01 | IS d4-9(S)-HODE | 295.2 | 171.1 |  |
| 55 | 15(16)-EpODE | 11.09 | IS d4-12(13)-EpOME | 293.2 | 275.2 |  |
| 56 | 17(18)-EpETE | 11.19 | IS d4-12(13)-EpOME | 317.2 | 259.2 |  |
| 57 | 15-HETE | 11.21 | IS d8-12(S)-HETE | 319.2 | 219.2 |  |
| 58 | 13-KODE | 11.27 | IS d4-9(S)-HODE | 293.2 | 179.1 |  |
| 59 | 9(10)-EpODE | 11.28 | IS d4-12(13)-EpOME | 293.2 | 275.2 |  |
| 60 | 17-HDoHE | 11.32 | IS d8-12(S)-HETE | 343.3 | 281.2 |  |
| 61 | 15-HpETE screen | 11.48 | IS d8-12(S)-HETE | 335.2 | 113.1 |  |
| 62 | 13-HpODE screen | 11.48 | IS d4-9(S)-HODE | 311.2 | 179.1 |  |
| 63 | 9-HpODE screen | 11.88 | IS d4-9(S)-HODE | 311.2 | 185.2 |  |
| 64 | 12(13)-EpODE | 11.48 | IS d4-12(13)-EpOME | 293.2 | 183.2 |  |
| 65 | 15-KETE | 11.55 | IS d8-12(S)-HETE | 317.2 | 273.2 |  |
| 66 | 14-HDoHE | 11.62 | IS d8-12(S)-HETE | 343.3 | 281.2 |  |
| 67 | 11-HETE | 11.64 | IS d8-12(S)-HETE | 319.2 | 167.1 |  |
| 68 | 14(15)-EpETE | 11.64 | IS d4-12(13)-EpOME | 317.2 | 247.2 |  |
| 69 | 9-KODE | 11.71 | IS d4-9(S)-HODE | 293.2 | 185.2 |  |
| 70 | d8-12(S)-HETE | 11.78 | IS CUDA | 327.2 | 184.2 |  |
| 71 | 11(12)-EpETE | 11.80 | IS d4-12(13)-EpOME | 317.2 | 167.3 |  |
| 72 | 12-HETE | 11.88 | IS d8-12(S)-HETE | 319.2 | 179.1 |  |
| 73 | 8-HETE | 12.02 | IS d8-12(S)-HETE | 319.2 | 155.1 |  |
| 74 | 9-HETE | 12.23 | IS d8-12(S)-HETE | 319.2 | 167.1 |  |
| 75 | 12-HpETE screen | 12.00 | IS d8-12(S)-HETE | 335.2 | 153.1 |  |
| 76 | 5-HpETE screen | 13.18 | IS d8-5(S)-HETE | 335.2 | 155.1 |  |
| 77 | d8-5(S)-HETE | 12.49 | IS CUDA | 327.2 | 116.1 |  |
| 78 | 19(20)-EpDPE | 12.57 | IS d4-12(13)-EpOME | 343.3 | 281.2 |  |
| 79 | 5-HETE | 12.58 | IS d8-5(S)-HETE | 319.2 | 115.1 |  |
| 80 | d4-12(13)-EpOME | 12.65 | IS CUDA | 299.2 | 198.1 |  |
| 81 | 12(13)-EpOME | 12.74 | IS d4-12(13)-EpOME | 295.2 | 195.2 |  |
| 82 | 14(15)-EpETrE | 12.83 | IS d4-12(13)-EpOME | 319.2 | 219.2 |  |
| 83 | 4-HDoHE | 12.88 | IS d8-5(S)-HETE | 343.3 | 281.2 |  |
| 84 | 16(17)-EpDPE | 12.96 | IS d4-12(13)-EpOME | 343.5 | 273.5 |  |
| 85 | 9(10)-EpOME | 12.97 | IS d4-12(13)-EpOME | 295.2 | 171.1 |  |
| 86 | 5-KETE | 13.28 | IS d8-5(S)-HETE | 317.2 | 203.2 |  |
| 87 | 11(12)-EpETrE | 13.31 | IS d4-12(13)-EpOME | 319.2 | 167.1 |  |
| 88 | 8(9)-EpETrE | 13.51 | IS d4-12(13)-EpOME | 319.2 | 155.1 |  |
| 89 | 10-Nitrolinoleate | 13.79 | IS d17-10-Nitrooleate | 324.3 | 277.2 |  |
| 90 | 9_10-EpO | 14.37 | IS 10_11-DHHep | 297.3 | 279.2 |  |
| 91 | d17-10-Nitrooleate | 14.60 | IS CUDA | 343.2 | 307.5 |  |
| 92 | 10-Nitrooleate | 14.7 | IS d17-10-Nitrooleate | 326.2 | 279.5 |  |
| 93 | 9-Nitrooleate | 14.80 | IS d17-10-Nitrooleate | 326.2 | 308.2 |  |
| 94 | 9_10-DiHHex | 15.21 | IS 10_11-DHHep | 287.2 | 227.5 |  |
| 95 | d8-Arachidonic Acid | 15.68 | IS CUDA | 311.3 | 267.1 |  |
| * - Analytes were separated under conditions described in Table I. Collision-induced dissociation was performed with nitrogen |  |  |  |  |  |  |
| at a pressure of 2.3 mTorr. Scheduled mass spectral multiple reaction monitoring was utilized to identify analytes. |  |  |  |  |  |  |
| † - Internal Standards (ISTD) - Analytes were corrected for recoveries of listed surrogates. 1-Cyclohexylureido,3-dodecanoic acid |  |  |  |  |  |  |
| (CUDA) and 1-Phenyl 3-Hexadecanoic Acid Urea (PHAU) were introduced immediately prior to analysis and used to quantify |  |  |  |  |  |  |
| surrogate recoveries. |  |  |  |  |  |  |
| †† - Compounds labeled as "screen" are compounds for which we did not have calibration standards. The |  |  |  |  |  |  |
| compounds were identified based on retention time and transition (Da) and produced qualitative data. |  |  |  |  |  |  |